Tera term to Tera term iam using windows non internet pc

Got it — you want Tera Term to Tera Term communication between two Windows PCs without internet, using UART. I’ll give you a clear, practical setup from scratch so you can actually make it work.

🔌 1. What you need

- 2 Windows PCs (no internet needed ✅)

- 2 USB-to-Serial adapters (or built-in COM ports)

- Null modem cable (very important ❗ — normal cable won’t work)

- Tera Term installed on both PCs

🔗 2. Physical Connection

6

- Connect like this:

  - PC1 → USB-to-Serial → Null Modem Cable → USB-to-Serial → PC2

- Why null modem?

  - It cross connects TX ↔ RX, enabling communication

⚙️ 3. Find COM Ports (Both PCs)

- Open Device Manager

- Expand Ports (COM & LPT)

- Note COM numbers

  - Example:

    - PC1 → COM3

    - PC2 → COM4

🖥️ 4. Open Tera Term on both PCs

- Launch Tera Term

- Select:

  - Serial

  - Choose correct COM port

⚙️ 5. Configure Serial Settings (IMPORTANT)

Set same values on both PCs:

- Baud rate: 115200

- Data bits: 8

- Parity: None

- Stop bits: 1

- Flow control: None

👉 Path in Tera Term:

Setup → Serial Port

🔁 6. Enable Local Echo (Optional)

- Go to:

Setup → Terminal → Local echo ON

- This helps you see what you type

💬 7. Test Communication

👉 Now type on PC1
👉 You should see it on PC2 instantly

And vice versa 🎉

⚠️ Common Problems (Most people get stuck here)

1. Nothing is showing

- Wrong cable (must be null modem ❗)

- COM port mismatch

- Baud rate mismatch

2. Garbage characters

- Different baud rates

- Wrong parity or stop bits

3. Port not opening

- Another app using COM port (close PuTTY, Arduino IDE, etc.)

🔥 Pro Tip (for your UART voice project)

Once this works, you can:

- Replace manual typing with your C++ program output

- Use same COM port settings in code

- Send encoded audio data through UART

🎯 If you want next step

I can give you:

- Full C++ UART send/receive code (Windows COM port)

- Or your voice over UART (mic + headphones setup)

- Or even a diagram + viva explanation

Just tell me 👍

How to send and receive any files using tera term

You can send files between two PCs using Tera Term over your UART connection — but here’s the reality first:

👉 It’s not like Bluetooth or USB file copy
👉 You must use file transfer protocols (XMODEM / YMODEM / ZMODEM)
👉 Both sides must coordinate (one sends, one receives)

📁 Method 1: Using XMODEM (Simple & Works Offline)

🔄 Steps (PC → PC transfer)

🟢 On RECEIVER PC (very important: start here)

5

- Open Tera Term

- Go to:

File → Transfer → XMODEM → Receive

- Choose:

  - File name (where to save)

  - Click Open

- Now it is waiting for sender

🔵 On SENDER PC

5

- Go to:

File → Transfer → XMODEM → Send

- Select your file

- Click Open / Send

🎉 Done!

- File will transfer over UART

- You’ll see progress bar

⚡ Method 2: YMODEM (Better for bigger files)

Same steps, just choose:

File → Transfer → YMODEM → Send/Receive

👉 Advantages:

- Faster than XMODEM

- Supports batch files

🚀 Method 3: ZMODEM (Best, but limited support)

File → Transfer → ZMODEM

👉 Faster + auto resume
👉 But sometimes not enabled depending on setup

⚙️ IMPORTANT SETTINGS (Must match both PCs)

- Baud rate: 115200 (or higher if stable)

- Data: 8N1

- Flow control: None

⚠️ Common Issues (and fixes)

❌ Transfer not starting

- Receiver didn’t start first
✔ Always start Receive first

❌ Transfer very slow

- XMODEM is slow by nature
✔ Use YMODEM

❌ File corrupted

- Baud mismatch

- Noise in cable
✔ Reduce speed (9600 / 57600)

❌ Nothing happens

- Wrong cable (must be null modem)

- COM port wrong

🔥 Pro Insight (for your project)

Since you're doing UART + voice/data project, this teaches you:

- How raw data packets move

- How error checking works

- Base for your own protocol in C++

If you want next level:

- I can show how to send files using your own C++ code (no XMODEM)

- Or build a mini file transfer app over UART like WhatsApp but serial 😄

Just tell me 👍

After this without using tera term i need to send and receive using simple socket python program using uart

You can’t use a socket program directly over UART the way you do with TCP/IP. Sockets belong to networking (IP stack), while UART is just a raw byte stream.

👉 So the correct approach is:

- Use pyserial for UART

- Then build your own “socket-like” send/receive protocol on top of it

Think of it like:
UART = pipe → you design rules → behaves like sockets

🧠 How your setup will work

PC1 (Python Sender)  ←UART→  PC2 (Python Receiver)

- No internet needed ✅

- Works exactly like Tera Term file transfer, but controlled by Python

⚙️ 1. Install library (both PCs)

(If no internet → copy wheel file manually)

pip install pyserial

📤 2. Sender Code (Python – send file via UART)

import serial import time import os ser = serial.Serial('COM3', 115200, timeout=1) filename = "test.txt" # Send file name first ser.write((filename + "\n").encode()) time.sleep(1) # Send file size filesize = os.path.getsize(filename) ser.write((str(filesize) + "\n").encode()) time.sleep(1) # Send file data with open(filename, "rb") as f: while True: data = f.read(1024) if not data: break ser.write(data) print("File sent successfully") ser.close()

📥 3. Receiver Code (Python – receive file)

import serial ser = serial.Serial('COM4', 115200, timeout=1) # Receive filename filename = ser.readline().decode().strip() # Receive filesize filesize = int(ser.readline().decode().strip()) with open("received_" + filename, "wb") as f: received = 0 while received < filesize: data = ser.read(1024) if data: f.write(data) received += len(data) print("File received successfully") ser.close()

🔁 4. How to run

- Connect PCs via null modem cable

- Match settings:

  - Baud: 115200

  - Same COM ports as before

- Run:

  - First → Receiver script

  - Then → Sender script

⚠️ Important limitations (don’t ignore)

❌ Not a real socket

- No built-in:

  - Retransmission

  - Error correction

  - Packet ordering

❌ Risk of corruption

UART is not reliable like TCP

🔥 Upgrade it (make it “socket-like”)

If you want next-level (good for interview 🔥):

Add:

- ACK/NACK system

- Packet numbering

- Checksum (CRC)

Then it becomes:
👉 “Custom transport protocol over UART”

🎯 Interview explanation (use this line)

“Sockets require an IP stack, so over UART I used PySerial and implemented a custom file transfer protocol with framing, metadata, and streaming to simulate socket-like communication.”

🚀 If you want next step

I can help you build:

- ✅ Full reliable UART protocol (like TCP mini version)

- ✅ Voice + file transfer combo system

- ✅ Diagram + viva explanation (very useful for interviews)

Just tell me 👍

1st and 2nd point

Got it — you want both upgrades:

1️⃣ ACK/NACK system
2️⃣ Packet numbering (sequence control)

This is exactly how you turn your UART program into a mini TCP-like reliable system 🔥

🧠 Concept (Simple Understanding)

Instead of sending raw data, we send:

[SEQ][DATA][CHECKSUM]

- SEQ → packet number

- DATA → actual bytes

- CHECKSUM → detect errors

Receiver will:

- Verify checksum

- Send back:

  - ACK ✅ (correct)

  - NACK ❌ (wrong → resend)

📤 Sender Code (with ACK + Sequence)

import serial import time ser = serial.Serial('COM3', 115200, timeout=1) filename = "test.txt" # Send filename ser.write((filename + "\n").encode()) time.sleep(1) seq = 0 with open(filename, "rb") as f: while True: data = f.read(64) if not data: break checksum = sum(data) % 256 packet = bytes([seq]) + data + bytes([checksum]) while True: ser.write(packet) ack = ser.read(3) if ack == b'ACK': print(f"Packet {seq} ACK") seq = (seq + 1) % 256 break else: print(f"Resending packet {seq}") ser.write(b'EOF') print("File sent successfully") ser.close()

📥 Receiver Code (with ACK + Validation)

import serial ser = serial.Serial('COM4', 115200, timeout=1) filename = ser.readline().decode().strip() with open("received_" + filename, "wb") as f: expected_seq = 0 while True: packet = ser.read(66) # 1(seq)+64(data)+1(checksum) if packet == b'EOF': break if len(packet) < 2: continue seq = packet[0] data = packet[1:-1] checksum = packet[-1] calc_checksum = sum(data) % 256 if seq == expected_seq and checksum == calc_checksum: f.write(data) ser.write(b'ACK') print(f"Received packet {seq}") expected_seq = (expected_seq + 1) % 256 else: ser.write(b'NACK') print(f"Error in packet {seq}") print("File received successfully") ser.close()

🔁 How it works (flow)

6

- Sender sends packet (SEQ 0)

- Receiver checks:

  - If correct → sends ACK

  - If wrong → sends NACK

- Sender:

  - ACK → next packet

  - NACK → resend same packet

👉 This is called Stop-and-Wait Protocol

⚠️ Important Improvements (optional but powerful)

🔹 Add timeout handling

If no ACK:

ser.timeout = 2

🔹 Add packet size control

- Use smaller chunks (32–128 bytes) for stability

🔹 Add start/end markers

Instead of raw packet:

[START][SEQ][DATA][CHECKSUM][END]

🎯 Interview Gold Line

“I implemented a reliable UART protocol using Stop-and-Wait with sequence numbers, checksum validation, and ACK/NACK retransmission to ensure data integrity without TCP.”

🚀 If you want next level

I can extend this into:

- Sliding window protocol (faster than stop-and-wait)

- Full voice + file hybrid system

- GUI file transfer app (like mini AirDrop over UART 😄)

Just tell me 👍

Where can I find pyserial for uart and how can I build my own socket -like send/receive protocol

You’re on the right track — this is exactly how people move from “using tools” to building their own communication system.

Let’s split it cleanly:

📦 1️⃣ Where to get PySerial (offline + online)

🔹 What it is

pyserial is the standard Python library to access UART/COM ports.

✅ Option A — If you have internet (quick)

pip install pyserial

✅ Option B — No internet (your case)

6

Steps:

- On another PC (with internet):

  - Go to:
👉 https://pypi.org/project/pyserial/#files

  - Download .whl file (example):

  - pyserial-3.5-py2.py3-none-any.whl

- Copy it using USB → to your offline PC

- Install:

pip install pyserial-3.5-py2.py3-none-any.whl

🔍 Verify installation

python -c "import serial; print('OK')"

🧠 2️⃣ How to build your own “socket-like” protocol

Let’s be honest:
👉 UART ≠ sockets
👉 You are basically creating your own TCP

🧱 Step-by-step architecture

🔹 Layer idea (like real networking)

Application Layer → Your File/Voice App Transport Layer → Your Protocol (ACK, SEQ, etc.) Physical Layer → UART (PySerial)

🔹 Core features you MUST build

1. Framing (start/end of packet)

<START> | SEQ | LENGTH | DATA | CHECKSUM | <END>

Example:

b'\x02' + seq + length + data + checksum + b'\x03'

2. Packet Numbering (SEQ)

- Prevent duplicate packets

- Maintain order

3. ACK / NACK system

- Receiver → sends ACK or NACK

- Sender → resend if needed

4. Checksum (error detection)

checksum = sum(data) % 256

5. Timeout + Retransmission

- If no ACK → resend

🔄 3️⃣ Full Communication Flow

6

Sender:

- Break file → chunks

- Add header (SEQ, checksum)

- Send packet

- Wait for ACK

- Retry if needed

Receiver:

- Detect START/END

- Extract packet

- Validate checksum

- Send ACK/NACK

- Store data

🧪 4️⃣ Minimal “Socket-like” Example Design

Instead of:

socket.send(data)

You build:

send_packet(data) wait_for_ack()

🔥 5️⃣ Real-world naming (for interviews)

What you’re building is:

- Stop-and-Wait ARQ Protocol

- Custom Transport Layer over UART

🎯 Strong interview line

“Since UART doesn’t support sockets, I implemented a custom transport protocol using PySerial with framing, sequencing, checksum validation, and ACK/NACK retransmission to ensure reliable communication.”

⚠️ Common mistakes (avoid these)

- ❌ No framing → data mixing

- ❌ Large packets → data loss

- ❌ No timeout → program hangs

- ❌ No checksum → corrupted files

🚀 If you want next level (recommended)

I can give you:

- ✅ Full modular protocol code (clean + professional)

- ✅ Sliding window (faster than stop-and-wait)

- ✅ Voice + file transfer combined system

- ✅ Block diagram + viva explanation (super helpful)

Just tell me 👍

Any videos regarding this

Here are some good, relevant YouTube videos that will help you understand exactly what you’re doing (PySerial + UART + building your own protocol).

🎥 1️⃣ Beginner – PySerial Basics (MUST WATCH FIRST)

👉 What you’ll learn:

- How to open COM port in Python

- Send & receive data using UART

- Basic setup (exactly like your project)

This is your foundation video.

🎥 2️⃣ PySerial Setup + Working Example

👉 Why this is useful:

- Quick understanding of functions

- read(), write(), timeouts

- Clean explanation (no confusion)

🎥 3️⃣ Practical Serial Communication (Real Hardware)

👉 Even though it uses Arduino:

- Concept is SAME as PC ↔ PC UART

- Shows real data flow

- Helps you visualize communication

🎥 4️⃣ Another Simple COM Port Example

👉 Focus:

- COM port configuration

- Serial parameters (baud, parity, etc.)

🧠 How to connect these videos to YOUR goal

After watching:

Step 1 (from videos)

- Learn:

  - serial.Serial()

  - .write()

  - .read()

👉 This is your UART layer

Step 2 (your upgrade)

Add:

- Packet format

- ACK/NACK

- Sequence number

👉 This becomes your socket-like protocol

🔥 Important (don’t skip this insight)

These videos teach:

“How to send data”

But YOU are building:

“How to send data reliably like TCP”

That’s the difference 🔥

🎯 Suggested learning order (perfect flow)

- Video 1 → basics

- Video 2 → functions

- Video 3 → real communication

- Then → implement your protocol (the code I gave earlier)

🚀 If you want next step

I can give you:

- 🎯 Full project-level code (clean + modular like real software)

- 🎯 GUI file transfer app (resume, progress bar)

- 🎯 Viva explanation + diagrams (for interview)

Just tell me 👍

Top of Form

Bottom of Form