Gemini

Tera Term File Transfer Non-Internet PCs

https://gemini.google.com/share/a48ead4779eb

Created with Fast 22 April 2026 at 23:38Published on 22 April 2026 at 23:43

Tera term to Tera term how send and receive a file on windows non internet pcs

Transferring files between two non-internet Windows PCs using Tera Term is a classic, reliable method. Since you aren't using a network, you will need a Null Modem Cable (RS-232) or two USB-to-Serial adapters connected back-to-back with a null modem adapter.

1. Physical Connection

Connect the two PCs via their COM ports. If your computers don't have DB9 serial ports (which most modern ones don't), use USB-to-Serial adapters.

- PC A USB → Serial Adapter → Null Modem Cable → Serial Adapter → PC B USB.

2. Tera Term Configuration

You must ensure both PCs are "speaking the same language" by matching their serial port settings. Open Tera Term on both machines and follow these steps:

- Select Serial and choose the correct COM port.

- Go to Setup > Serial port...

- Match these settings exactly on both PCs:

  - Baud rate: 115200 (highest stable speed)

  - Data: 8 bit

  - Parity: none

  - Stop: 1 bit

  - Flow control: hardware (or none, if hardware isn't supported by your cable)

- Click New setting.

Pro Tip: Type a few characters in the terminal window. If you see them appear on the other computer's screen, your connection is successful.

3. Sending and Receiving

While there are several protocols (XMODEM, YMODEM, etc.), ZMODEM is generally the best choice because it is fast and can automatically start the transfer on the receiving end.

Step A: Prepare the Receiver (PC B)

- On the receiving PC, go to File > Transfer > ZMODEM > Receive.

- A small window will pop up waiting for the file.

Step B: Send the File (PC A)

- On the sending PC, go to File > Transfer > ZMODEM > Send...

- Select the file you want to transfer.

- Click Open.

The transfer window will show a progress bar. Once it hits 100%, the file will be located in the Tera Term installation folder on the receiving PC (usually C:\Program Files (x86)\teraterm) unless you changed the default "File transfer directory" in the settings.

Troubleshooting Tips

- Garbage Characters: If you see weird symbols instead of the text you type, your Baud rates likely don't match. Double-check Setup > Serial port.

- Transfer Fails Immediately: Try a slower Baud rate (e.g., 9600 or 57600) or switch the protocol to XMODEM, though XMODEM requires you to manually name the file on the receiving end first.

- COM Port Not Found: Check Device Manager on Windows to see which COM port number was assigned to your USB-to-Serial adapter.

Iam using one laptop and another is windows

Since both of your machines are Windows (one laptop, one PC), the process is identical for both. The most important part is the physical cable: you cannot use a standard USB charging cable. You need a USB-to-USB Null Modem Cable or two USB-to-Serial adapters connected with a null modem adapter.

Here is exactly how to set up the transfer:

1. Identify your COM Ports

Before opening Tera Term, you need to know which "gate" the cable is using on each computer.

- Plug your cable into both computers.

- On both computers, right-click the Start button and select Device Manager.

- Expand the Ports (COM & LPT) section.

- Look for "USB Serial Port" and note the number (e.g., COM3 on the laptop and COM4 on the PC).

2. Match the Tera Term Settings

Open Tera Term on both machines. If they aren't perfectly synced, the data will look like "garbage" text.

- New Connection: Select Serial and pick your COM port.

- Serial Setup: Go to Setup > Serial port... and enter these exact values on both:

  - Baud rate: 115200 (Speed)

  - Data: 8 bit

  - Parity: none

  - Stop: 1 bit

  - Flow control: hardware

- Test: Type "Hello" on the laptop. If it appears on the PC screen, you are ready to send files.

3. Sending the File (The "ZMODEM" Method)

ZMODEM is the best protocol because it's "smart"—it handles the file name and the save location automatically.

| On the Receiving Computer | On the Sending Computer |
| --- | --- |
| 1. Go to File > Transfer > ZMODEM > Receive. | 1. Go to File > Transfer > ZMODEM > Send... |
| 2. A small status box will appear, waiting. | 2. Browse and select your file. |
|  | 3. Click Open. |

Where does the file go? By default, the file is saved in the Tera Term installation folder. To find it quickly:

- Go to File > Change directory... on the receiving machine before you start the transfer to pick an easier spot, like your Desktop.

Comparison of Transfer Protocols

If ZMODEM fails, you can try these alternatives within the File > Transfer menu:

| Protocol | Best For... | Note |
| --- | --- | --- |
| ZMODEM | Everything | Fastest and easiest; handles filenames automatically. |
| YMODEM | Large Files | Good backup if ZMODEM isn't working. |
| XMODEM | Small Files | Very basic. You must manually type the filename on the receiver. |
| Kermit | Poor Connections | Very slow, but will work even if the cable connection is "noisy." |

Why isn't it working?

- Wrong Cable: If you are using a standard USB male-to-male cable, it won't work and could actually damage your USB ports. It must be a "Null Modem" or "Laplink" style cable.

- Driver Issue: If the COM ports don't show up in Device Manager, you may need to install the "FTDI" or "CH340" drivers for your specific cable using a USB drive from a computer that does have internet.

Google Privacy PolicyOpens in a new windowGoogle Terms of ServiceOpens in a new windowYour privacy and Gemini AppsOpens in a new window

Gemini may display inaccurate info, including about people, so double-check its responses.