You can send files between two PCs using Tera Term over your UART connection — but here’s the reality first:

👉 It’s not like Bluetooth or USB file copy
👉 You must use file transfer protocols (XMODEM / YMODEM / ZMODEM)
👉 Both sides must coordinate (one sends, one receives)

# 📁 Method 1: Using XMODEM (Simple & Works Offline)

## 🔄 Steps (PC → PC transfer)

### 🟢 On RECEIVER PC (very important: start here)

5

- Open Tera Term

- Go to:

File → Transfer → XMODEM → Receive

- Choose: You can send files between two PCs using Tera Term over your UART connection — but here’s the reality first:

- 👉 It’s not like Bluetooth or USB file copy

- 👉 You must use file transfer protocols (XMODEM / YMODEM / ZMODEM)

- 👉 Both sides must coordinate (one sends, one receives)

- ________________________________________

- 📁 Method 1: Using XMODEM (Simple & Works Offline)

- 🔄 Steps (PC → PC transfer)

- 🟢 On RECEIVER PC (very important: start here)

- 5

- Open Tera Term

- Go to:

- File → Transfer → XMODEM → Receive

- Choose:

- File name (where to save)

- Click Open

- Now it is waiting for sender

- ________________________________________

- 🔵 On SENDER PC

- 5

- Go to:

- File → Transfer → XMODEM → Send

- Select your file

- Click Open / Send

- ________________________________________

- 🎉 Done!

- File will transfer over UART

- You’ll see progress bar

- ________________________________________

- ⚡ Method 2: YMODEM (Better for bigger files)

- Same steps, just choose:

- File → Transfer → YMODEM → Send/Receive

- 👉 Advantages:

- Faster than XMODEM

- Supports batch files

- ________________________________________

- 🚀 Method 3: ZMODEM (Best, but limited support)

- File → Transfer → ZMODEM

- 👉 Faster + auto resume

- 👉 But sometimes not enabled depending on setup

- ________________________________________

- ⚙️ IMPORTANT SETTINGS (Must match both PCs)

- Baud rate: 115200 (or higher if stable)

- Data: 8N1

- Flow control: None

- ________________________________________

- ⚠️ Common Issues (and fixes)

- ❌ Transfer not starting

- Receiver didn’t start first

- ✔ Always start Receive first

- ________________________________________

- ❌ Transfer very slow

- XMODEM is slow by nature

- ✔ Use YMODEM

- ________________________________________

- ❌ File corrupted

- Baud mismatch

- Noise in cable

- ✔ Reduce speed (9600 / 57600)

- ________________________________________

- ❌ Nothing happens

- Wrong cable (must be null modem)

- COM port wrong

- ________________________________________

- 🔥 Pro Insight (for your project)

- Since you're doing UART + voice/data project, this teaches you:

- How raw data packets move

- How error checking works

- Base for your own protocol in C++

- ________________________________________

- If you want next level:

- I can show how to send files using your own C++ code (no XMODEM)

- Or build a mini file transfer app over UART like WhatsApp but serial 😄

- Just tell me 👍

  - File name (where to save)

  - Click Open

- Now it is waiting for sender

### 🔵 On SENDER PC

5

- Go to:

File → Transfer → XMODEM → Send

- Select your file

- Click Open / Send

### 🎉 Done!

- File will transfer over UART

- You’ll see progress bar

# ⚡ Method 2: YMODEM (Better for bigger files)

Same steps, just choose:

File → Transfer → YMODEM → Send/Receive

👉 Advantages:

- Faster than XMODEM

- Supports batch files

# 🚀 Method 3: ZMODEM (Best, but limited support)

File → Transfer → ZMODEM

👉 Faster + auto resume
👉 But sometimes not enabled depending on setup

# ⚙️ IMPORTANT SETTINGS (Must match both PCs)

- Baud rate: 115200 (or higher if stable)

- Data: 8N1

- Flow control: None

# ⚠️ Common Issues (and fixes)

### ❌ Transfer not starting

- Receiver didn’t start first
✔ Always start Receive first

### ❌ Transfer very slow

- XMODEM is slow by nature
✔ Use YMODEM

### ❌ File corrupted

- Baud mismatch

- Noise in cable
✔ Reduce speed (9600 / 57600)

### ❌ Nothing happens

- Wrong cable (must be null modem)

- COM port wrong

# 🔥 Pro Insight (for your project)

Since you're doing UART + voice/data project, this teaches you:

- How raw data packets move

- How error checking works

- Base for your own protocol in C++

If you want next level:

- I can show how to send files using your own C++ code (no XMODEM)

- Or build a mini file transfer app over UART like WhatsApp but serial 😄

Just tell me 👍